|  | CHBE 376: Computer Flowsheeting & Unit Operation Design |
| --- | --- |

CHBE 376

Instructor: Jonathan Verrett

Term Project – Aniline

Submitted by:

Name:		Muhamad Nabil Alhanif

Student ID:	76787712

Signature:	_______________________

Contents

1.	Process Description (a short introduction, suggest no more than 1 page)	3

2.	Block Flow Diagram (to show your overall process)	4

3.	Components	5

4.	Property method	5

5.	Reactor Setup	6

6.	Analysis [eg. design spec, sensitivity analysis, calculator block, recycle stream, optimization])	8

7.	Column Setup	8

8.	Analysis	11

9.	References	13

Update the table above by right clicking and selecting “Update Field”

## Process Description (a short introduction, suggest no more than 1 page)

Our simulation will focus on the production of aniline via the liquid-phase catalytic hydrogenation of nitrobenzene. This process is comprised of one reaction, as follows:

C6H5NO2 + 3 H2 -> C6H5NH2 + 2H2O (1)

Reaction 1 is a highly exothermic reaction carried out at 0.15 – 1 MPa pressure and 150 - 250 C, combining hydrogen gas with liquid or gas-phase nitrobenzene [1], [2]. The hydrogenation of benzene accounts for 85% of global aniline production [2]. Liquid-phase hydrogenation is selected for this project due to its lower operating temperature and pressure, reducing safety risks. This reaction is typically catalysed by copper/silicon, nickel, or platinum/palladium catalyst. At low temperatures, palladium catalyst is preferred due to its high selectivity [3].

## Block Flow Diagram (to show your overall process)

This one was made using Visio, but feel free to use whichever program to create it, or even can be done by hand and scanned.

Figure 1: Block Flow Diagram for Cumene process

## Components

The Aniline process is comprised of one reaction [1]:

C6H5NO2 + 3 H2 -> C6H5NH2 + 2H2O (1)

The major components relevant to the simulation are listed in table 1 below.

Table 1: Component names and formulas relevant to the simulation

| Name | Formula |
| --- | --- |
| Hydrogen | H2 |
| Nitrobenzene | C6H5NO2 |
| Aniline | C6H5NH2 |
| Water | H2O |

## Property method

Operating condition for the process lies around 0.15 – 1 MPa and 150 - 250°C [2], although Turakova et al. suggests running at 2 – 4 MPa. The reaction species comprise of an amine (aniline) and a nitroaromatic compound (nitrobenzene) in liquid phase. It also contains hydrogen gas, which is insoluble in atmospheric pressure. Given this, the system would be classified as a chemical system at high pressure. A suitable overall property method would be an activity coefficient model such as SR-POLAR, SRK, PSRK and others, with addition of Henry’s method to model the hydrogen gas.

## Reactor Design

The reactor setup will facilitate the hydrogenation of nitrobenzene in liquid phase via metal catalysts.

C6H5NO2 + 3 H2 -> C6H5NH2 + 2H2O (1)

For the process, nitrobenzene is dissolved in solvent such as methanol, isopropanol, or ethanol [3]. Additionally, solvent helps maintain reaction temperature. This is important as reaction (1) is highly exothermic [3].

Within the reactor, both mass transfer and reaction kinetic affects the reaction rate. On mass transfer, three phases are identified: dissolution of H2 in liquid, diffusion of dissolved H2 and reactant to catalyst surface, and intra-particle diffusion to the active sides [4]. Mass transfer limitation can be mitigated by: increasing temperature, increasing mixing, and reducing catalyst size [5], [6]. For simulation purposes, effect of mass transfer limitation will be ignored.

Modelling reaction kinetics, it has been proposed that hydrogenation of nitrobenzene can be modelled as a Langmuir-Hinshelwood-Hougen-Watson (LHHW) kinetics [3], [7]. This is due to the non-competitive chemisorption of nitrobenzene and hydrogen gas to catalyst surface [3]. As common in Aspen, the catalyst is not accounted for in components, rather being a part of the rate law.

LHHW reaction kinetic for aniline is presented as follows [7]:

dAndt=k1K1CNBnK2pH2m1+K1CNBn+K2pH2m2

Where constants for 2 catalysts are shown in Table 2.

Table 2: Reaction, adsorption constants and reaction orders for nitrobenzene hydrogenations. Adapted from [7]

| Catalyst | K1 (1/mol) | K2 (1/mol) | k1 (L Bar)/(s mol) | n | M |
| --- | --- | --- | --- | --- | --- |
| Ni(18.4)-HPS | 1287 | 524 | 30619 | 0.87 | 1.12 |
| Pd(0.5)-Ni(17.9)HPS | 7563 | 9293 | 59496 | 1.05 | 1.18 |

Note to instructors: I’m still trying to understand how it relates to the amount of catalysts, as the units from equation above doesn’t contain anything about the catalyst. Source [3] offers a similar LHHW kinetics, but without the numerical values.

This reactor setup will focus on the cumene (C) production step, which was reaction (1) shown previously and is shown again for reference below. This is typically produced using a solid phosphoric acid catalyst (supported on alumina) or an aluminum chloride catalyst [6]. A common side reaction noted for this system is the formation of diisopropylbenzenes (DIPB), shown in reaction (4). These can also form further products including tri- or tetraisopropylbenzenes, however these further side reactions will simply be considered to be taken into account with the formation of DIPBs.

C6H6 + C3H6  → C9H12  (1)

C9H12 + C3H6 → C12H18 (4)

In many processes a mixture of propylene (P) and propane is mixed with fresh and recycled benzene (B). The propane is a common contaminant in propylene, but is generally not reactive. For this simulation we will assume a 95% propylene purity with the remainder being propane, this purity has been used in economic studies of the cumene process [6]. This mixture is typically preheated to temperatures ranging from 180 °C to 200 °C and fed into a fixed bed reactor at 550 psig. The reaction is exothermic and typically reactor temperatures will range from 350 °C to 410 °C [7]. Benzene is typically in large excess in these reactions in order to help moderate the exothermic reaction. Early technologies fed in benzene to propylene at ratio of 8:1, but processes have improved to now feed 5:1 with interbed injection. The greater the benzene feed, the more costly the separation of benzene for recycle, so there is a tradeoff between controlling the reaction and getting good conversion and the cost of separation. For this simulation we will aim to produce 1% of the total phenol used in the world, so 70,000 tonnes of phenol per year, this translates to 750 tonne-moles/yr. To start in our simulation, we will assuming an overall process efficiency of 80% and that the plant runs for 350 days per year. This means we need to feed 2650 kmol/day or 110 kmol/h of propylene into the process. This will be used as a starting basis. We will use a feed ratio of 5:1 benzene to propylene.

For this system, a plug flow reactor (RPLUG) will be used to simulate this reaction step. Kinetic data provided as an example by Turton et al. describes rates for the two relevant reactions in this process [7]. As is common with Aspen, the catalyst is not accounted for in the components and is taken into account in the rate law.

Reaction 1: r1=k1CPCB where k1=3.5×104e-24.9 kcal/mol*T[K]

Reaction 4: r4=k4CPCC where k1=2.9×106e-35.08 kcal/mol*T[K]

In the rate laws above the concentrations are in mol/L, and the overall rate in mol/(gcat s). The reactor will initially be set up at a specified temperature of 380 °C, which could be adjusted in the future to optimize the process. Initially before setting up the kinetic reactor, an equilibrium reactor (REQUIL) was setup to see equilibrium conversion. This converted 87% of the propylene to cumene, with a small amount left uncoverted (1.5%) and the remainder going to the formation of DIPB. Literature values state the reactors can achieve 80% conversion, and this will be the aim in this simulation [7]. We will use kinetic reactor setup specifications from Luyben which is based on kinetics from Turton et al. which include catalyst tubes of 0.0732m in diameter and 6m in length packed with 5mm diameter spherical catalyst particles with a density of 1600 kg/m3 and a voidage of 0.5 [8]. Luyben uses 342 tubes for a flow of 110 kmol/h of a feed propylene stream, and we will keep this same number of tubes for this initial simulation. All reactants and reactions are assumed to be in the vapour phase. With the above specifications, conversion of propylene to cumene is 98.4 %. This may be high, but is deemed acceptable for the start of simulation as this will likely change when adding more realistic heat transfer and recycle streams. This conversion is higher than equilibrium conversion, but this may be due to the side reaction being less favored kinetically. Concentration profiles over the reactor are shown in figure 2 below.

Figure 2: Concentration profiles over the kinetic reactor

NOTE – For your reactor setup you may use more screenshots to show what you inputted rather than text. See the full simulation example PDF handed out at the start of the term. Also for this reactor I had to go through 3 different sets of kinetic data to find some data that made sense and gave reasonable values. I say this to show you how difficult it is to obtain kinetic data that gives reasonable results. If you are having trouble setting up your reactor that is completely normal, make a note of your sources and perhaps keep these in an appendix for reference. Always be careful of units, especially activation energies. The bkp for this simulation is also available for you to look over if that is helpful.

## Reactor Analysis [eg. design spec, sensitivity analysis, calculator block, recycle stream, optimization])

## Column Design

## Column Analysis

## Heat Exchanger Design

## Heat Exchanger Analysis

## Discussion/reflection

## Next steps

## References

[1]	T. Kahl, K.-W. Schröder, F. R. Lawrence, W. J. Marshall, H. Höke, and R. Jäckh, ‘Aniline’, in Ullmann’s Encyclopedia of Industrial Chemistry, John Wiley & Sons, Ltd, 2011. doi: 10.1002/14356007.a02_303.pub2.

[2]	R. Gao, C. Peng, and J.-J. Zou, ‘Selective Catalytic Hydrogenation of Nitroarenes to Anilines’, in Industrial Arene Chemistry, John Wiley & Sons, Ltd, 2023, pp. 1479–1524. doi: 10.1002/9783527827992.ch50.

[3]	M. Turáková, T. Salmi, K. Eränen, J. Wärnå, D. Yu. Murzin, and M. Králik, ‘Liquid phase hydrogenation of nitrobenzene’, Applied Catalysis A: General, vol. 499, pp. 66–76, Jun. 2015, doi: 10.1016/j.apcata.2015.04.002.

[4]	X. Duan, J. Yin, M. Huang, P. Wang, and J. Zhang, ‘Hydrogenation kinetics of halogenated nitroaromatics over Pt/C in a continuous Micro-packed bed reactor’, Chemical Engineering Science, vol. 251, p. 117483, Apr. 2022, doi: 10.1016/j.ces.2022.117483.

[5]	N. Frikha, E. Schaer, and J.-L. Houzelot, ‘Methodology of multiphase reaction kinetics and hydrodynamics identification: Application to catalyzed nitrobenzene hydrogenation’, Chemical Engineering Journal, vol. 124, no. 1, pp. 19–28, Nov. 2006, doi: 10.1016/j.cej.2006.08.012.

[6]	G. Li, J. Ma, X. Peng, and C. Lv, ‘Kinetics Approach for Hydrogenation of Nitrobenzene to p-Aminophenol in a Four-Phase System’, Asian Journal of Chemistry, vol. 23, no. 3, pp. 1001–1005, 2011.

[7]	R. Brovko, L. Mushinsky, A. Latypova, M. Sulman, V. Matveeva, and V. Doluda, ‘Evaluation of Nitrobenzene Hydrogenation Kinetic Particularities over Mono and Bimetallic Ni Containing Hypercrosslinked Polystyrene’, Chemical Engineering Transactions, vol. 86, pp. 883–888, Jun. 2021, doi: 10.3303/CET2186148.