|  | CHBE 376: Computer Flowsheeting & Unit Operation Design |
| --- | --- |

CHBE 376

Instructor: Jonathan Verrett

Term Project – [Title here – eg Cumene]

Submitted by:

Name:		_______________________

Student ID:	_______________________

Signature:	_______________________

Contents

1.	Process Description (a short introduction, suggest no more than 1 page)	3

2.	Block Flow Diagram (to show your overall process)	4

3.	Components	5

4.	Property method	5

5.	Reactor Setup	6

6.	Analysis [eg. design spec, sensitivity analysis, calculator block, recycle stream, optimization])	8

7.	Column Setup	8

8.	Analysis	11

9.	References	13

Update the table above by right clicking and selecting “Update Field”

## Process Description (a short introduction, suggest no more than 1 page)

Our simulation will focus on the cumene process for the production of phenol and acetone. The process is widely used and was estimated to have generated approximately 7 million tonnes of phenol in 2003 alone [1]. The Cumene Process is comprised of three reactions carried out at varying conditions [2]:

C6H6 + C3H6 → C9H12 (1)

C9H12 + O2 → C9H12O2 (2)

C9H12O2 → C6H6O + C3H6O (3)

Reaction 1 is a synthesis reaction carried out at 30 atm pressure and 250°C combining vapour-phase benzene and propylene to produce cumene, and is commonly catalyzed with phosphoric acid [3]. Reaction 1 is conducted in the gas phase in order to achieve higher conversion than in the liquid phase [4]. In reaction 2 the oxygen from air is used to oxidize cumene and form cumene hydroperoxide; the cumene hydroperoxide autocatalyzes the reaction, and the reacting conditions are 5 atm and 95°C in order to ensure the reaction occurs in the liquid phase to keep the peroxides stable [3]. The third and final reaction is a decomposition reaction cleaving cumene hydroperoxide into phenol and acetone, which takes place also in the liquid-phase at 5 atm and 70°C, and is typically catalyzed with a low concentration sulfuric acid solution [5]. Catalysis will be ignored in this process simulation for each of the reactions.

## Block Flow Diagram (to show your overall process)

This one was made using Visio, but feel free to use whichever program to create it, or even can be done by hand and scanned.

Figure 1: Block Flow Diagram for Cumene process

## Components

The Cumene Process is comprised of three reactions carried out at varying conditions [2]:

C6H6 + C3H6 → C9H12 (1)

C9H12 + O2 → C9H12O2 (2)

C9H12O2 → C6H6O + C3H6O (3)

The major components relevant to the simulation are listed in table 1 below.

Table 1: Component names and formulas relevant to the simulation

| Name | Formula |
| --- | --- |
| Oxygen | O2 |
| Propylene | C3H6 |
| Benzene | C6H6 |
| Cumene | C9H12 |
| Cumene hydroperoxide | C9H12O2 |
| Phenol | C6H6O |
| Acetone | C3H6O |

## Property method

Conditions in the process vary from 1 to 30 atm and 20 to 250°C. There are some non-hydrocarbon components namely oxygen, cumene hydroperoxide, phenol and acetone. Given this, this would be classified as a chemical system at high pressures. A suitable overall property method would be an equation of state method such as SR-POLAR, PRWS, RKSWS etc.

## Reactor Design

This reactor setup will focus on the cumene (C) production step, which was reaction (1) shown previously and is shown again for reference below. This is typically produced using a solid phosphoric acid catalyst (supported on alumina) or an aluminum chloride catalyst [6]. A common side reaction noted for this system is the formation of diisopropylbenzenes (DIPB), shown in reaction (4). These can also form further products including tri- or tetraisopropylbenzenes, however these further side reactions will simply be considered to be taken into account with the formation of DIPBs.

C6H6 + C3H6  → C9H12  (1)

C9H12 + C3H6 → C12H18 (4)

In many processes a mixture of propylene (P) and propane is mixed with fresh and recycled benzene (B). The propane is a common contaminant in propylene, but is generally not reactive. For this simulation we will assume a 95% propylene purity with the remainder being propane, this purity has been used in economic studies of the cumene process [6]. This mixture is typically preheated to temperatures ranging from 180 °C to 200 °C and fed into a fixed bed reactor at 550 psig. The reaction is exothermic and typically reactor temperatures will range from 350 °C to 410 °C [7]. Benzene is typically in large excess in these reactions in order to help moderate the exothermic reaction. Early technologies fed in benzene to propylene at ratio of 8:1, but processes have improved to now feed 5:1 with interbed injection. The greater the benzene feed, the more costly the separation of benzene for recycle, so there is a tradeoff between controlling the reaction and getting good conversion and the cost of separation. For this simulation we will aim to produce 1% of the total phenol used in the world, so 70,000 tonnes of phenol per year, this translates to 750 tonne-moles/yr. To start in our simulation, we will assuming an overall process efficiency of 80% and that the plant runs for 350 days per year. This means we need to feed 2650 kmol/day or 110 kmol/h of propylene into the process. This will be used as a starting basis. We will use a feed ratio of 5:1 benzene to propylene.

For this system, a plug flow reactor (RPLUG) will be used to simulate this reaction step. Kinetic data provided as an example by Turton et al. describes rates for the two relevant reactions in this process [7]. As is common with Aspen, the catalyst is not accounted for in the components and is taken into account in the rate law.

Reaction 1: r1=k1CPCB where k1=3.5×104e-24.9 kcal/mol*T[K]

Reaction 4: r4=k4CPCC where k1=2.9×106e-35.08 kcal/mol*T[K]

In the rate laws above the concentrations are in mol/L, and the overall rate in mol/(gcat s). The reactor will initially be set up at a specified temperature of 380 °C, which will be adjusted in the future to optimize the process. Initially before setting up the kinetic reactor, an equilibrium reactor (REQUIL) was setup to see equilibrium conversion. This converted 87% of the propylene to cumene, with a small amount left uncoverted (1.5%) and the remainder going to the formation of DIPB. Literature values state the reactors can achieve 80% conversion, and this will be the aim in this simulation [7]. We will use kinetic reactor setup specifications from Luyben which is based on kinetics from Turton et al. which include catalyst tubes of 0.0732m in diameter and 6m in length packed with 5mm diameter spherical catalyst particles with a density of 1600 kg/m3 and a voidage of 0.5 [8]. Luyben uses 342 tubes for a flow of 110 kmol/h of a feed propylene stream, and we will keep this same number of tubes for this initial simulation. All reactants and reactions are assumed to be in the vapour phase. With the above specifications, conversion of propylene to cumene is 98.4 %. This may be high, but is deemed acceptable for the start of simulation as this will likely change when adding more realistic heat transfer and recycle streams. This conversion is higher than equilibrium conversion, but this may be due to the side reaction being less favored kinetically. Concentration profiles over the reactor are shown in figure 2 below.

Figure 2: Concentration profiles over the kinetic reactor

NOTE – For your reactor setup you may use more screenshots to show what you inputted rather than text. See the full simulation example PDF handed out at the start of the term. Also for this reactor I had to go through 3 different sets of kinetic data to find some data that made sense and gave reasonable values. I say this to show you how difficult it is to obtain kinetic data that gives reasonable results. If you are having trouble setting up your reactor that is completely normal, make a note of your sources and perhaps keep these in an appendix for reference. Always be careful of units, especially activation energies. The bkp for this simulation is also available for you to look over if that is helpful.

## Reactor Analysis [eg. design spec, sensitivity analysis, calculator block, recycle stream, optimization])

In the case of the reactor, literature values suggest a conversion of roughly 80% of the propylene entering the reactor. We will use a design spec to achieve this, manipulating our reactor length to get the desired conversion. In this case our design spec provide a reactor length of 2.22 metres, which is quite a bit smaller than our initial 6m. This length has been incorporated into the reactor and the design spec has been left in the simulation but deactivated.

## Column Design

One separation we may focus on in this process for setting up a column is the separation of benzene for recycle from cumene and other heavier side-products. Figure 3 shows the reaction and separation units from a sample cumene production process.

Figure 3: Reaction and Separation units for Cumene production. Where units are: R1 Alkylation reactor; R2 Transalkylation reactor; C1 Depropanizer; C2 Benzene column; C3 Cumene column; C4 Polyisopropylbenzene (PIPB) column [1]

In the case of our simulation we will use the stream leaving our reactor as a starting point for separation, this stream consists of the following components and their respective flows, shown in table 2 ordered by their boiling points at atmospheric pressure.

Table 2:

| Component Name | Flow rate out of reactor (kmol/h) | Normal Boiling Point (°C) |
| --- | --- | --- |
| Propylene | 20.9 | -47.7 |
| Propane | 466.6 | -42.0 |
| Benzene | 83.1 | 80.0 |
| Cumene | 5.5 | 152.4 |
| Diisopropylbenzenes | 0.2 | 210.1 |

For our column, we will assume all propylene and propane has been removed given their much lower boiling points, leaving a stream of benzene, cumene and diisopropylbenzenes to separate. In this case the light key component will be a benzene stream for recycle to the reactor and the heavy key will be cumene. Diisopropylbenzenes are a non-key component and will be expected to leave with the cumene. In the case of our final cumene product, the maximum benzene concentration is listed to be a maximum of 0.001 wt% [1]. We will use this as a weight percent to achieve for our separation. The process overall can achieve a cumene yield of 99% based on benzene [1]. Given this, we will assume a higher recovery than 99% in our column which would allow for losses of benzene to side reaction or purging. We will initially assume a recovery of 99.99% benzene, and fine tune our final column to achieve the desired benzene concentration in the bottoms being no more than 0.001 wt%. We will assume the recovery of cumene as a starting guess is also 99.99%.

To obtain an initial estimate of the column specifications a DSTWU will be used. The entering stream contain the flow rates of benzene, cumene and diisopropylbenzenes as outlined in table 1. This feed stream will be at 1 atm pressure and at the bubble point. The estimates used for the DSTWU will include: reflux ratio 1.35 times the minimum reflux ratio. The column will operate at 1 atm with a total condenser.

The initial results from the DSTWU provided the following results:

Actual reflux ratio: 0.230

Number of stages: 27

Feed stage: 13

Distillate to feed fraction: 0.936

Given this predicts a large number of stages and a low reflux ratio, a reflux ratio vs number of stages analysis was done going from 10 stages (minimum number of stages) to 40 stages. The plot obtained is shown in figure 4. The DTWU was re-run with a reflux ratio of 1.5 times the minimum reflux ratio and gives a column with the following results:

Actual reflux ratio: 0.255

Number of stages: 23

Feed stage: 11

Distillate to feed fraction: 0.936

Figure 4: Reflux ratio vs the number of stages from DSTWU block

Given these results, a higher reflux ratio corresponding to 1.5 times the minimum reflux ratio will be used as a starting point for the Radfrac column as an initial estimate in order to minimize the number of column stages while only increasing the reflux ratio slightly. Using those criteria the Radfrac column provides the results shown in figure 5 in terms of stream flows.

Figure 5: Stream flows from Radfrac column with initial setup

The bottoms stream enriched in cumene contains 7.5x10-9 benzene in terms of mass fraction, which will meet our purity specifications for the cumene stream in terms of benzene concentration (0.001 wt%). The benzene recovery of the column to the distillate is very high, over 99.999%, meeting our desired recovery of 99.99%. Given this, our column may be overdesigned in terms of what would be needed. The column will be optimized in the analysis section that follows.

## Column Analysis

Figure 6 presents a plot of the k-values at each stage, which represent the mole fraction of a compound in the vapour divided by its mole fraction in the liquid. We see there is little change over a number of stages. As a starting point, we will remove 5 stages above and 6 stages below the feed stage. Meaning the column will now have 12 stages with 6 being the feed stage.

Figure 6: k-values for a variety of stages for the initial Radfrac column setup

With this new column arrangement the bottoms stream contains 0.076 wt% benzene, which will not meet our purity specifications of (0.001 wt%). There is still a small amount of diisopropylbenzenes in the stream and if we remove these from our total mass, the weight fraction of benzene is 0.0794 wt%. The benzene recovery of the column to the distillate is still sufficiently high at over 99.992%, meeting our desired recovery of 99.99%. In order to get our purity specification in terms of benzene weight percent in the bottoms stream to our desired level, the reflux ratio will be manipulated to achieve this.

[DOCUMENT TO BE CONTINUED WITH RESULTS OF ANALYSIS]

## Heat Exchanger Design

The cumene reactor requires a feed of propylene [6]. This mixture is typically preheated to temperatures ranging from 180 °C to 200 °C and fed into a fixed bed reactor at 550 psig. The reactor feed is a stream of 110 kmol/hr consisting of 95 mol% propylene with the remainder being propane. Assuming it is stored at ambient temperature (assumed to be 20°C) and high pressure a heat exchanger will be designed to heat this stream to 190 °C. The utility that will be used to perform this heating will be high pressure steam at 250°C as this will provide a sufficient driving force for heat transfer. Medium pressure steam at 175°C, being less expensive, would not be hot enough to heat the stream to its desired temperature of 190°C. The steam pressure could be altered to provide a temperature closer to the desired 190°C, however, for the purpose of this simulation, the default high pressure steam temperature of 250°C will be used.

The flow of steam is found using a HEATER block setting high pressure steam as a utility. This results in a steam flow of 1737 kg/hr. A HEATX block is set up with saturated steam (vapour fraction of 1) at 250 C being fed into the hot inlet with a flow of 1737 kg/hr. The propylene mixture is fed in the cold side at 20 C and the HEATX block is set to alter this cold stream temperature to 190 C. The defaults are left in place for the HEATX block, using a shortcut model, with countercurrent heat exchange and no pressure drop. The phases present are condensing on the hot side of the exchanger and liquid, boiling and vapour on the cold side of the exchanger. Overall heat transfer coefficients (U) for the components present are estimated for these conditions. The propylene is assumed to have similar properties to propane and an overall heat transfer coefficient between steam and propane in a vapourizer is found to be in the range of 200-300 BTU/(F*ft2*hr) [9]. Liquid is assumed to have a higher heat transfer coefficient as a denser phase and a value of 300 BTU/(F*ft2*hr) is used. For vapour, a heat transfer coefficient of 200 BTU/(F*ft2*hr) is used. For boiling, a heat transfer coefficient of 250 BTU/(F*ft2*hr) is used. With these heat transfer coefficients, the resulting heat exchanger area obtained is 4.65 square metres.

## Heat Exchanger Analysis

An analysis is performed to assess the impact of the assumptions of heat transfer coefficients on heat exchanger area. A sensitivity analysis is set up varying the overall heat transfer coefficient from 200 to 300 BTU/(F*ft2*hr). The heat exchanger is altered such that it has a single specified heat transfer coefficient and this is what the sensitivity analysis manipulates. Results from the analysis are shown in figure 7. Heat exchanger area varies from roughly 5.75 m2 to 3.8 m2. This represents a 24% increase or a 19% decrease in heat exchanger area at the lowest and highest overall heat transfer coefficients respectively. This showcases the potential variability in the heat exchanger design and the need for a good estimate for the heat transfer coefficient. Other ways of estimating the heat transfer coefficient may include experimental tests or from heat and mass transfer fundamentals, and these may be used to check the heat transfer coefficient used in the design.

Figure 7: Impact of overall heat transfer coefficient (U) on heat exchanger area

## References

[1] M. Weber, M. Weber, and M. Kleine-Boymann. “Acetone” in Ullmann’s Encyclopedia of Industrial Chemistry, 2004. Wiley-VCH.

[2] A. Streitwieser, and C.H. Heathcock. “Introduction to Organic Chemistry,” 1992. MacMillan.

[3] University of York, “Phenol,” 2014. [Online]. Available:

http://www.essentialchemicalindustry.org/chemicals/phenol.html

[4] Engineers Guide, “Cumene production flow sheet and Process description,” 2009. [Online].

Available: http://enggyd.blogspot.ca/2009/09/cumene-production-flow-sheet.html

[5] University of California – Davis, “Naming and Preparing Benzenols.” [Online]. Available:

http://sasc-specialists.ucdavis.edu/jim/118C/BenzenolFrmCumeneHydroperox.pdf

[6] S. Y. Hwang and S. S. Chen, “Cumene,” in Kirk-Othmer Encyclopedia of Chemical Technology, American Cancer Society, 2010, pp. 1–10.

[7] R. Turton, R. C. Bailie, W. B. Whiting, and J. A. Shaeiwitz, Analysis, synthesis and design of chemical processes. Apendix C: Project 6. Pearson Education, 2008.

[8] W. L. Luyben, “Design and Control of the Cumene Process,” Ind. Eng. Chem. Res., vol. 49, no. 2, pp. 719–734, Jan. 2010, doi: 10.1021/ie9011535.

[9] W. D. Seider, D. R. Lewin, J. D. Seader, S. Widagdo, R. Gani, and K. M. Ng Product and process design principles: synthesis, analysis, and evaluation, 4th edition. Section 12.3 Heat Transfer Coefficients and Pressure Drop. John Wiley & Sons, 2017.